EEL 5606 Assignment 3

Trade Study Document

Simulation-Based Autonomous UGV for Indoor Shipyard Maintenance Corridor Inspection

Chief Systems Engineer Trade Study

| Field | Value |
| --- | --- |
| Student | Justin Kwarteng |
| Course | EEL 5606 Introduction to Mobile Robotics and Unmanned Systems |
| Project Type | Simulation-only ROS 2 Humble / Gazebo-Ignition / optional RViz |
| Baseline Vehicle | 2WD differential-drive rover with caster wheel, encoder-style odometry, IMU, 2D range sensing, and simulated health reporting |
| Trade Study Area | Range-sensing subsystem for obstacle detection and avoidance |

Submitted: June 2026

Document purpose: identify feasible technical approaches, screen them against must-have mission criteria, compare acceptable options with a weighted decision matrix, and select the best sensing approach for the course prototype.

Core assumption: The course deliverable remains a simulation-only prototype. Physical products are used only as market-research anchors for realism, future transferability, and cost awareness.

# 1. Project Description and Design Reference Mission

This project designs and demonstrates a simulation-based autonomous unmanned ground vehicle (UGV) for remote inspection of an indoor shipyard or port maintenance corridor after a suspected obstruction, alarm, storm-related issue, or hazardous condition. The vehicle is intended to enter the defined corridor before personnel, follow a predefined inspection route, detect obstacles or blocked paths, avoid obstacles when a safe maneuver is available, and report mission status to an operator station. The purpose is to reduce unnecessary human exposure while improving the speed, repeatability, and documentation quality of initial inspections.

The prototype is implemented entirely in software using ROS 2 Humble for autonomy logic, command handling, telemetry, and logging. Gazebo/Ignition simulates the robot, corridor, drivetrain, sensor outputs, obstacles, and system state. RViz may be used to visualize odometry, range data, robot pose, and mission status. The baseline robot is a two-wheel differential-drive rover with a caster wheel, encoder-style odometry, an IMU, range sensing, and simulated battery/system-health reporting. The route is predefined; full SLAM-based mapping is reserved for future work.

Figure 1. Design reference mission flow for the indoor maintenance corridor inspection mission.

Table 1. Mission Measures of Effectiveness (MOEs).

| ID | Measure | Definition | Target |
| --- | --- | --- | --- |
| MOE-1 | Inspection mission success rate | Standardized test runs in which the UGV reaches the endpoint or enters safe-stop without collision. | >= 90% |
| MOE-2 | Obstacle-response effectiveness | Route obstacles detected and handled without collision. | >= 95% |
| MOE-3 | Inspection completion time | Elapsed time from START command to endpoint arrival or safe-stop report. | <= 6 minutes |
| MOE-4 | Inspection-data reporting completeness | Runs in which the operator receives a complete mission record with progress, obstacle events, health, elapsed time, and final outcome. | >= 95% |

Table 2. Verification scenarios used to judge mission effectiveness.

| Scenario | Expected Behavior |
| --- | --- |
| Nominal route | Operator sends one START command; UGV follows the predefined route and reaches the endpoint. |
| Avoidable obstacle | UGV detects a path obstacle, slows or stops, executes a simple avoidance maneuver, resumes route, and logs the event. |
| Blocked route | UGV determines no safe alternate path is available, stops safely, and notifies the operator. |
| Sensor/timeout contingency | If a sensor stream is invalid or the mission exceeds allowed time, the UGV enters safe-stop and reports the fault. |

# 2. Project Work Breakdown Structure

The WBS below organizes the project around the major products needed to demonstrate the reference mission in simulation. The structure also supports requirements traceability, subsystem verification, and integration planning.

Table 3. Project work breakdown structure.

| WBS | Element | Scope |
| --- | --- | --- |
| 1.0 | Systems Engineering and Project Management | Requirements traceability, trade study, risk tracking, schedule control, documentation. |
| 1.1 | Requirements and MOE/MOP management | Maintain mission requirements, verification matrix, and design assumptions. |
| 1.2 | Trade study and design decisions | Evaluate subsystem alternatives and select the course baseline. |
| 2.0 | Simulation Environment | Gazebo/Ignition world, corridor route, obstacles, robot spawn, ROS-Gazebo bridge. |
| 2.1 | Corridor/world model | Known indoor corridor or warehouse-style route with endpoint and obstacle zones. |
| 2.2 | Robot model | URDF/SDF vehicle with base, wheels, caster, links, joints, collisions, inertials, and plugins. |
| 3.0 | Mobility Subsystem | 2WD differential-drive vehicle, caster wheel, cmd_vel interface, odometry output. |
| 3.1 | Differential-drive actuation | Gazebo differential-drive plugin and ROS 2 velocity command interface. |
| 3.2 | Mobility performance tuning | Tune speed, acceleration, turning behavior, stopping distance, and collision geometry. |
| 4.0 | Navigation and State Estimation | Encoder-style odometry, IMU orientation, endpoint tracking, route progress estimation. |
| 5.0 | Sensing Subsystem | Range sensor suite, IMU stream, simulated health/battery status, sensor validity checks. |
| 5.1 | Obstacle-detection sensor | Selected trade-study item: simulated 2D LiDAR or alternate range approach. |
| 6.0 | Autonomy and Safety Software | START command, route follower, obstacle-response logic, safe-stop logic, timeout handling. |
| 7.0 | Telemetry and Logging | Operator status interface, route progress, sensor health, obstacle events, mission summary. |
| 8.0 | Verification and Validation | Nominal, avoidable-obstacle, blocked-route, and sensor/timeout contingency tests. |

# 2.1 Key Subsystems, Performance Requirements, and Baseline Approaches

The subsystem requirements below trace to the prior requirements document and are stated in terms that can be verified in Gazebo/ROS 2 demonstrations. The baseline technical approach is the lowest-risk approach that has worked for similar indoor mobile robots or mobile-robot simulations.

Table 4. Subsystem requirements, baseline approach, and verification method.

| Subsystem | Requirement Trace | Performance Requirement | Baseline Technical Approach | Verification |
| --- | --- | --- | --- | --- |
| Mission Management | REQ-SYS-001, REQ-SYS-002 | Begin autonomous inspection after one START command; complete nominal mission within 6 minutes. | Finite-state mission manager with states such as IDLE, INSPECTING, OBSTACLE_RESPONSE, SAFE_STOP, COMPLETE, and FAULT. | Demonstration and timed test |
| Mobility Controller | REQ-MOB-001 | Support commanded forward travel from 0.20 to 0.35 m/s during inspection. | 2WD differential drive with caster wheel, Gazebo differential-drive plugin, /cmd_vel input, /odom output. | Velocity and route tests |
| Navigation / Odometry | REQ-NAV-001, REQ-NAV-002 | Publish encoder-style odometry at >=10 Hz; finish nominal route within 0.50 m of endpoint. | Wheel-odometry baseline with optional IMU heading monitoring; no full SLAM required for course prototype. | Topic-rate test and endpoint measurement |
| Range Sensor Suite | REQ-SEN-001 | Detect a solid obstacle in planned path at >=0.75 m. | Baseline range sensor model attached to front/top of rover; compared formally in trade study. | Obstacle-detection range test |
| IMU Sensor | REQ-SEN-002 | Publish simulated IMU orientation data at >=10 Hz during mission. | Gazebo IMU sensor and ROS 2 topic output used for heading/system-health monitoring. | Topic-rate inspection/test |
| Safety Manager | REQ-SAF-001 | Stop within 0.35 m after unsafe-path command at 0.30 m/s. | Reactive safety node overrides /cmd_vel with zero velocity and records safe-stop event. | Stopping-distance test |
| Obstacle-Avoidance Software | REQ-AUT-001, REQ-AUT-002 | Initiate stop or avoidance within 2 seconds; enter safe-stop and notify operator if no safe alternate route is found within 10 seconds. | Reactive obstacle sectors: stop, evaluate left/right clearance, attempt simple bypass, resume route if safe. | Avoidable and blocked obstacle tests |
| Telemetry / Logging | REQ-COM-001, REQ-LOG-001 | Report route progress, mission state, obstacle events, health, and simulated battery at >=1 Hz; generate mission summary. | ROS 2 status topic plus console/file log; optional RViz visualization for pose and sensor data. | Telemetry-rate test and mission-log inspection |

# 3. Selected Trade Study Area and Rationale

The formal trade study focuses on the range-sensing subsystem used for obstacle detection and avoidance. This area was selected because it has direct impact on mission success, collision avoidance, stopping distance, route resumption, operator reporting, and the complexity of the ROS 2/Gazebo integration.

The sensor choice directly traces to MOE-2 obstacle-response effectiveness and MOP-2 obstacle-detection range.

The sensor topic becomes an input to autonomy and safety logic, so a poor choice can ripple into route following, avoidance, telemetry, and verification.

The current development path is already centered on adding and validating LiDAR/range data in Gazebo and ROS 2, so this trade study supports an immediate design decision.

A simulation-only project still benefits from realistic market research because the selected simulated sensor should have a plausible physical equivalent for future work.

There are meaningfully different alternatives: simple point range sensors, multi-sensor arrays, 2D LiDAR, RGB-D depth sensing, camera-only vision, and fusion packages. These differ strongly in capability, implementation risk, cost, and schedule impact.

Trade-study decision statement:

Select the obstacle-detection sensor architecture that best supports reliable indoor corridor obstacle detection, simple ROS 2/Gazebo implementation, low schedule risk, mission telemetry, and future extension to mapping or navigation.

# 4. Metrics and Must-Have Screening Criteria

Initial screening uses must-have criteria before weighted scoring. Any option that fails a must-have is removed from the final matrix, even if it has attractive features in other areas.

Table 5. Must-have screening criteria in priority order.

| Priority | Criterion | Minimum Requirement | Trace |
| --- | --- | --- | --- |
| 1 | Obstacle detection distance | Must detect a solid obstacle in the planned path at >=0.75 m. | REQ-SEN-001, MOP-2, MOE-2 |
| 2 | Timely response support | Must provide data usable by control software quickly enough to start stop/avoidance within <=2 seconds and support safe stopping within <=0.35 m at 0.30 m/s. | REQ-AUT-001, REQ-SAF-001, MOP-3, MOP-6 |
| 3 | ROS 2/Gazebo compatibility | Must be implementable as a simulated sensor topic that can feed autonomy, safety, logging, and optional RViz visualization. | REQ-COM-001, REQ-LOG-001 |
| 4 | Indoor corridor suitability | Must work for predefined-route indoor corridor inspection and not require full SLAM, heavy ML, or large environment changes for the course prototype. | Mission assumptions |
| 5 | Course schedule feasibility | Must be implementable and testable within the remaining assignment/project timeline using the existing ROS 2/Gazebo setup. | Project constraint |
| 6 | Future physical realism | Should have a plausible low-cost physical equivalent that can fit within the future prototype budget planning range of about $600-$900. | Planning estimate |

Table 6. Weighted scoring metrics in priority order.

| Metric | Weight (%) | Meaning |
| --- | --- | --- |
| Obstacle detection reliability and coverage | 25 | How reliably the option detects solid obstacles early enough across the corridor path and nearby side-clearance region. |
| ROS 2/Gazebo integration and development risk | 20 | How directly the option maps to known Gazebo/ROS messages, plugins, and visualization tools. |
| Implementation effort and schedule | 15 | How quickly the option can be added, debugged, verified, and explained in the final course demonstration. |
| Navigation and avoidance usefulness | 15 | How much useful geometry the option gives to stop, pick an avoidance side, resume route, and log estimated obstacle location. |
| Cost and physical prototype realism | 10 | How plausible the option is for a future low-cost physical prototype. |
| Telemetry, visualization, and testability | 10 | How easy it is to inspect topic output, visualize data, and produce evidence for verification. |
| Future scalability | 5 | How well the option supports later upgrades such as SLAM, costmaps, localization, or more complex environments. |

Scoring scale: 5 = excellent, 4 = good, 3 = acceptable, 2 = weak, 1 = poor. Weighted score equals score/5 multiplied by the metric weight.

# 5. Technical Approaches Considered

The approaches below combine brainstorming with market research. For a simulation-only course project, the selected simulated sensor should still correspond to real sensor families that have credible future physical equivalents.

Table 7. Candidate sensing approaches.

| ID | Approach | Description / Market Anchor | Strengths | Concerns |
| --- | --- | --- | --- | --- |
| A | Front-only ultrasonic or IR range sensor | One forward distance sensor modeled as sonar/IR/ray range. Physical analogs include low-cost ultrasonic modules and IR distance sensors. | Very simple, low cost, easy to explain; enough for stop-before-hit behavior. | Limited field of view; poor side-clearance information; less useful for avoidance and obstacle location. |
| B | Multi-direction ultrasonic/IR range array | Three to five range sensors covering front, front-left, front-right, and side clearances. | Still low-cost and easy; better than front-only for simple avoidance side selection. | Sparse geometry; gaps between beams; still less useful for mapping, RViz, and costmap-style reasoning. |
| C | 2D scanning LiDAR | Simulated 2D ray/gpu_lidar sensor publishing scan ranges. Physical analogs include low-cost 360-degree 2D LiDAR such as RPLIDAR-class devices. | Strong obstacle coverage, direct range data, easy LaserScan-style visualization, strong fit for mobile robot navigation. | More data and setup complexity than point sensors; physical units cost more than ultrasonic sensors. |
| D | RGB-D depth camera | Depth camera producing depth image or point cloud. Physical analog: Intel RealSense D435-class active stereo camera. | Rich 3D information; useful for object shape and future perception work. | Overkill for 2D corridor inspection; more processing, larger data streams, more integration complexity. |
| E | Monocular RGB camera with computer vision | Single camera with image-processing or ML obstacle detection. | Low hardware cost and useful for human visual inspection if developed fully. | No direct range without additional assumptions; high algorithm risk; outside current scope. |
| F | 2D LiDAR with odometry/IMU fusion and health checks | LiDAR remains primary obstacle sensor; odometry and IMU are used for pose sanity, logging, and fault checks. | Most robust architecture and best future upgrade path; supports obstacle location estimates and health-aware behavior. | Higher software complexity and more integration/test burden than LiDAR alone. |

Market-research notes: Gazebo Sensors provides configurable sensor models and noise models for simulation [1]. Nav2 documentation describes sensors as core inputs to mapping, localization, perception, and costmap generation [2]. The RPLIDAR A1 is a representative low-cost 2D LiDAR family with 360-degree scanning and more than 8000 distance measurements per second [3]. HC-SR04-class ultrasonic modules are representative low-cost range sensors with approximately 2 cm to 400 cm range [4], while Sharp GP2Y0A21YK0F-class IR sensors are representative short-range analog distance sensors with a 10 cm to 80 cm range [5]. Intel RealSense D435-class cameras represent RGB-D depth options with approximately 0.3 m to 3 m operating range and 1280 x 720 depth resolution [6]. Raspberry Pi Camera Module 3 is a representative monocular camera option, but it does not provide direct range by itself [7].

# 6. Initial Screening and Pareto Discussion

Each candidate was screened against the must-have criteria. Options marked pass remain eligible for the weighted matrix. Marginal options may pass only if their limitations can be mitigated within the course scope.

Table 8. Must-have screening results.

| ID | Approach | Detect >=0.75 m | <=2 s response | ROS/Gazebo | Indoor fit | Schedule | Physical realism | Result | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Front-only ultrasonic/IR | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Meets minimum stop-before-collision behavior, but provides weak coverage for avoidance. |
| B | Multi-direction ultrasonic/IR array | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Acceptable low-cost backup; better than A for simple avoidance. |
| C | 2D scanning LiDAR | Pass | Pass | Pass | Pass | Pass | Pass | Pass | Strongest direct fit to obstacle detection, RViz visualization, and future navigation. |
| D | RGB-D depth camera | Pass | Pass | Pass | Pass | Marginal | Marginal | Pass | Capable but higher complexity than mission requires. |
| E | Monocular RGB camera + CV | Fail | Marginal | Pass | Fail | Fail | Pass | Fail | Removed: no direct range and too much perception risk for the course prototype. |
| F | 2D LiDAR + odom/IMU fusion | Pass | Pass | Pass | Pass | Marginal | Pass | Pass | High capability, but higher integration burden. |

Pareto frontier discussion:

Front-only ultrasonic/IR is on the low-cost/low-complexity frontier, but it is dominated in mission performance by the multi-direction range array and by 2D LiDAR.

The multi-direction range array is a practical backup option if the project were constrained to extremely low cost or if LiDAR integration failed late in the schedule.

2D LiDAR is Pareto strong because it provides high obstacle-detection capability with moderate implementation effort and excellent ROS/RViz testability.

RGB-D depth sensing provides richer data than required, but for this mission it is dominated by 2D LiDAR because it adds complexity without a matching mission benefit.

Monocular vision is screened out because the current project does not require onboard camera vision or ML, and monocular images do not directly provide obstacle range.

LiDAR plus odometry/IMU fusion is a high-performance frontier option, but its added integration effort is not necessary to satisfy Assignment 3 and the current simulation milestone.

# 7. Weighted Decision Matrix

The five options that passed initial screening were scored from 1 to 5. The weighted score is normalized to a 100-point scale.

Table 9. Raw option scores using 1-5 scale.

| ID | Option | Reliability / coverage 25% | ROS/Gazebo risk 20% | Effort / schedule 15% | Avoidance value 15% | Cost realism 10% | Testability 10% | Future scalability 5% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Front-only ultrasonic/IR | 2 | 4 | 5 | 2 | 5 | 3 | 2 |
| B | Multi-direction ultrasonic/IR array | 3 | 4 | 4 | 3 | 4 | 3 | 3 |
| C | 2D scanning LiDAR | 5 | 5 | 4 | 5 | 3 | 5 | 4 |
| D | RGB-D depth camera | 4 | 3 | 2 | 4 | 2 | 4 | 4 |
| F | 2D LiDAR + odom/IMU fusion | 5 | 4 | 2 | 5 | 3 | 5 | 5 |

Table 10. Weighted scores and ranking.

| Rank ID | Option | Reliability 25 | Integration 20 | Schedule 15 | Avoidance 15 | Cost 10 | Testability 10 | Future 5 | Total / 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C | 2D scanning LiDAR | 25.0 | 20.0 | 12.0 | 15.0 | 6.0 | 10.0 | 4.0 | 92.0 |
| F | 2D LiDAR + odom/IMU fusion | 25.0 | 16.0 | 6.0 | 15.0 | 6.0 | 10.0 | 5.0 | 83.0 |
| B | Multi-direction ultrasonic/IR array | 15.0 | 16.0 | 12.0 | 9.0 | 8.0 | 6.0 | 3.0 | 69.0 |
| D | RGB-D depth camera | 20.0 | 12.0 | 6.0 | 12.0 | 4.0 | 8.0 | 4.0 | 66.0 |
| A | Front-only ultrasonic/IR | 10.0 | 16.0 | 15.0 | 6.0 | 10.0 | 6.0 | 2.0 | 65.0 |

Base-case result: 2D scanning LiDAR ranks highest at 92/100. It meets every must-have criterion and provides the best balance between obstacle-detection performance, ROS 2/Gazebo implementation risk, RViz/testability, and future navigation value.

# 8. Sensitivity Analysis

The sensitivity analysis checks whether reasonable changes in assumptions would change the selected option. Three weighting scenarios were considered: base case, cost/schedule-dominant, and future-autonomy-dominant.

Table 11. Weighting sensitivity results.

| Scenario | Weights | 1st | 2nd | 3rd | 4th | 5th | Conclusion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Base case | Reliability 25, Integration 20, Schedule 15, Avoidance 15, Cost 10, Testability 10, Future 5 | C: 92.0 | F: 83.0 | B: 69.0 | D: 66.0 | A: 65.0 | C selected |
| Cost/schedule dominant | Reliability 20, Integration 15, Schedule 30, Avoidance 10, Cost 20, Testability 3, Future 2 | C: 85.6 | A: 76.6 | B: 73.0 | F: 71.0 | D: 57.0 | C still selected |
| Future-autonomy dominant | Reliability 30, Integration 15, Schedule 10, Avoidance 20, Cost 5, Testability 10, Future 10 | C: 94.0 | F: 89.0 | B: 67.0 | D: 70.0 | A: 57.0 | C still selected |

Interpretation: The result is robust within normal uncertainty. 2D LiDAR remains first even when schedule and cost are weighted much more heavily than in the base case. The outcome would change only under a stronger constraint than assumed, such as a rule forbidding LiDAR, a severe integration failure in the Gazebo/ROS bridge, or a physical-build budget that forces the project to use only a few dollar-level point sensors. Those changes are outside the current simulation-only project assumptions. The credible backup is the multi-direction ultrasonic/IR array because it preserves simple obstacle detection with lower cost and complexity, but it would reduce obstacle-location quality and future navigation value.

# 9. Selected Technical Option

Selected option: 2D scanning LiDAR as the primary simulated range sensor for obstacle detection and avoidance.

The selected approach is a Gazebo/Ignition 2D scanning LiDAR/ray sensor mounted on the UGV and bridged or published into ROS 2 as a scan topic for autonomy, safety, logging, and optional RViz visualization. This option best supports the reference mission because it provides direct range measurements across the front and surrounding area of the rover, enables simple obstacle-sector logic, supports verification of the 0.75 m detection requirement, and produces data that can be visualized and logged during test runs. It also has a realistic physical future-work path because low-cost 2D LiDAR units exist for hobbyist and research mobile robots.

Table 12. Implementation plan for the selected sensing approach.

| Area | Planned Implementation |
| --- | --- |
| Sensor model | Add a 2D ray/gpu_lidar sensor to the URDF/SDF model with a fixed frame such as lidar_link. |
| ROS topic | Publish or bridge simulated scan data to /scan and verify data with ros2 topic echo and RViz LaserScan display. |
| Suggested settings | Use a 180-360 degree horizontal scan, 0.12 m minimum range, 6-12 m maximum range, and approximately 10 Hz update rate for course testing. |
| Obstacle logic | Use a front-sector threshold: if any valid range in the path sector is less than the safety threshold, slow/stop, evaluate side clearance, and attempt a simple bypass if safe. |
| Safety behavior | If the scan is invalid, blocked, or no safe alternative exists within 10 seconds, enter SAFE_STOP and report the condition. |
| Verification | Run nominal route, avoidable obstacle, blocked route, and sensor/timeout contingency scenarios; record detection range, response time, stopping distance, and mission outcome. |
| Backup path | If LiDAR integration becomes unstable, switch to a multi-direction simulated range array for minimum obstacle detection and safe-stop demonstration. |

Final selection rationale: 2D LiDAR is the strongest choice for this project because it satisfies the must-have criteria, gives the autonomy software usable obstacle geometry, is easy to verify through /scan and RViz, stays within the simulation-only scope, and leaves a credible path to future SLAM, costmaps, and physical prototyping. The additional complexity compared with ultrasonic/IR sensors is justified by its large improvement in obstacle detection coverage and test evidence quality.

# References

[1] Gazebo Sensors documentation, Gazebo Sim. https://gazebosim.org/libs/sensors/ Accessed June 13, 2026.

[2] Nav2 documentation, 'Setting Up Sensors - Gazebo.' https://docs.nav2.org/setup_guides/sensors/setup_sensors_gz.html Accessed June 13, 2026.

[3] SLAMTEC, 'RPLIDAR A1.' https://www.slamtec.com/en/lidar/a1 Accessed June 13, 2026.

[4] ElecFreaks/SparkFun datasheet, 'Ultrasonic Ranging Module HC-SR04.' https://cdn.sparkfun.com/datasheets/Sensors/Proximity/HCSR04.pdf Accessed June 13, 2026.

[5] Sharp, 'GP2Y0A21YK0F Distance Measuring Sensor Unit' datasheet. https://global.sharp/products/device/lineup/data/pdf/datasheet/gp2y0a21yk_e.pdf Accessed June 13, 2026.

[6] Intel, 'Intel RealSense Depth Camera D435 Specifications.' https://www.intel.com/content/www/us/en/products/sku/128255/intel-realsense-depth-camera-d435/specifications.html Accessed June 13, 2026.

[7] Raspberry Pi, 'Camera Module 3.' https://www.raspberrypi.com/products/camera-module-3/ Accessed June 13, 2026.